Remote Patient Monitoring (RPM) System

Communication Architecture

1. Question

Design and explain the communication architecture of a Remote Patient Monitoring (RPM) system, covering its layers, protocols, and data flow from sensor to healthcare provider.

2. Definition

A Remote Patient Monitoring (RPM) Communication Architecture is the layered network and protocol framework that enables continuous or periodic acquisition of physiological data (e.g., ECG, SpO₂, blood pressure, glucose, temperature) from wearable or bedside sensors on a patient, and its reliable, secure, and timely transmission to a remote server or healthcare provider for storage, analysis, and clinical decision-making.

3. Description

The architecture is organized into four layers:

a) Sensing / Device Layer (Perception Layer)

- Wearable, implantable, or bedside biosensors (ECG, PPG, SpO₂, BP cuff, glucometer, temperature probe)

- Converts physiological signals into digital data

- Uses low-power microcontrollers (e.g., ARM Cortex-M, ESP32)

b) Gateway / Edge Layer

- Aggregates data from multiple sensors (via BLE, Zigbee, NFC)

- Performs edge processing: filtering, compression, anomaly pre-detection

- Devices: smartphone app, home hub, or dedicated IoT gateway

c) Network / Communication Layer

- Transmits data from gateway to cloud / hospital server

- Technologies: Wi-Fi, 4G/5G cellular, LoRaWAN (long-range, low-power), Ethernet (fixed hubs)

- Protocols: MQTT, CoAP, HTTP/HTTPS, WebSocket for real-time streaming

d) Application / Cloud Layer

- Cloud server or hospital EHR system stores and processes data

- Runs analytics and AI-based alerting, dashboards

- Sends alerts to clinicians via SMS, app notification, or EHR integration

- Ensures compliance with HIPAA / GDPR security standards

Data Flow

| Stage | Description |
| --- | --- |
| 1. Patient Sensor | ECG, SpO2, BP, glucose, temperature sensors acquire raw physiological signals. |
| 2. Short-Range Link | Bluetooth Low Energy (BLE) or Zigbee transmits data to a nearby gateway. |
| 3. Edge Gateway | Smartphone or home hub aggregates, timestamps, compresses, and encrypts data. |
| 4. Wide-Area Network | Wi-Fi / 4G-5G / LoRaWAN carries data from gateway to the internet. |
| 5. Cloud Server / Database | Stores data, runs analytics and AI-based anomaly detection. |
| 6. Clinician Dashboard / EHR | Visualizes trends, issues alerts, and integrates with hospital records. |
| 7. Feedback to Patient | Alerts, medication reminders, or care instructions sent back to the patient. |

4. Architecture Diagram

5. Formula

a) Nyquist Sampling Rate

fₛ ≥ 2 f_max

Minimum sampling rate to capture a signal without loss (e.g., ECG → fₛ ≥ 300 Hz).

b) Channel Capacity (Shannon-Hartley Theorem)

C = B log₂(1 + S/N)

C = channel capacity (bps), B = bandwidth (Hz), S/N = signal-to-noise ratio.

c) End-to-End Latency

T_total = T_sense + T_process + T_transmit + T_display

Sum of delays across the sensing-to-display pipeline.

d) Battery Life Estimation

L = C_battery / I_avg

C_battery = battery capacity (mAh), I_avg = average current drawn (mA).

6. Procedure

Step 1 — Requirement Analysis: Identify vital signs, sampling rate, and latency tolerance needed for monitoring.

Step 2 — Sensor & Edge Design: Select biosensors and low-power MCU; configure gateway to aggregate and encrypt data.

Step 3 — Network Selection: Choose BLE/Zigbee for short range and Wi-Fi/4G-5G/LoRaWAN for long range based on deployment.

Step 4 — Protocol & Cloud Setup: Implement MQTT/CoAP for transfer; deploy a HIPAA-compliant cloud backend with encryption (TLS/AES-256).

Step 5 — Analytics & Alerts: Apply threshold or ML-based anomaly detection to trigger clinician alerts via dashboard/EHR.

Step 6 — Testing & Validation: Validate latency, packet loss, battery life, and security under real-world conditions.

CODE:

% =========================================================================

% SIMULATION: Remote Patient Monitoring Communication Architecture

% Architecture: Sensor Nodes -> Wireless Channel -> IoT Gateway -> Cloud Server

% =========================================================================

clear; clc; close all;

%% 1. SYSTEM PARAMETERS & CONFIGURATION

fs = 100; % Sampling frequency (Hz)

duration = 10; % Simulation duration (seconds)

t = 0:1/fs:duration-1/fs; % Time vector

N = length(t);

num_patients = 3; % Number of patient sensor nodes

snr_dB = 15; % Channel Signal-to-Noise Ratio (dB)

%% 2. SENSOR NODE DATA GENERATION (Patient Terminals)

% Generate Synthetic Vital Signs for Patient Nodes

rng(42); % Set seed for reproducibility

% Node 1: Synthetic ECG Signal (~72 BPM with P-QRS-T components)

f_hr = 1.2; % Heart rate in Hz (~72 bpm)

ecg_clean = -0.15 * cos(2*pi*f_hr*t) ... % Baseline & P-wave

+ 1.2 * exp(-((mod(t, 1/f_hr) - 0.25)/0.02).^2) ... % QRS Complex

+ 0.25 * exp(-((mod(t, 1/f_hr) - 0.45)/0.08).^2); % T-wave

ecg_signal = ecg_clean + 0.05 * randn(1, N);

% Node 2: SpO2 Level (Pulse Oximetry around 98% with small fluctuations)

spo2_signal = 98 + 0.5 * sin(2*pi*0.1*t) + 0.2 * randn(1, N);

% Node 3: Body Temperature (in Celsius around 37.0 °C)

temp_signal = 37.0 + 0.1 * sin(2*pi*0.05*t) + 0.05 * randn(1, N);

%% 3. WIRELESS COMMUNICATION CHANNEL (Custom AWGN without Communications Toolbox)

add_awgn = @(sig, snr) sig + (sqrt((sum(abs(sig).^2)/length(sig)) / (10^(snr/10)))) * randn(size(sig));

ecg_tx = add_awgn(ecg_signal, snr_dB);

spo2_tx = add_awgn(spo2_signal, snr_dB);

temp_tx = add_awgn(temp_signal, snr_dB);

%% 4. IoT GATEWAY PROCESSING & ALERTS

% Gateway receives signals, filters noise, and checks for vital thresholds

% Digital Low-pass Filtering for ECG at Gateway

cutoff_freq = 35;

[b, a] = butter(4, cutoff_freq/(fs/2), 'low');

ecg_rx = filtfilt(b, a, ecg_tx);

% Moving Average Smoothing for SpO2 and Temperature

spo2_rx = movmean(spo2_tx, 10);

temp_rx = movmean(temp_tx, 10);

% Threshold Alert Logic

mean_spo2 = mean(spo2_rx);

mean_temp = mean(temp_rx);

fprintf('--- IoT GATEWAY HEALTH STATUS REPORT ---\n');

fprintf('Patient 1 Mean SpO2: %.2f%%\n', mean_spo2);

fprintf('Patient 1 Mean Temp: %.2f °C\n', mean_temp);

if mean_spo2 < 95

warning('ALERT: Low SpO2 level detected!');

else

fprintf('SpO2 Status: NORMAL\n');

end

if mean_temp > 37.5

warning('ALERT: High Temperature (Fever) detected!');

else

fprintf('Temperature Status: NORMAL\n');

end

%% 5. DATA VISUALIZATION (Central Monitoring Dashboard)

figure('Name', 'Remote Patient Monitoring Dashboard', 'NumberTitle', 'off');

% Plot ECG Signal

subplot(3, 1, 1);

plot(t, ecg_signal, 'k--', 'LineWidth', 1); hold on;

plot(t, ecg_rx, 'r', 'LineWidth', 1.2);

title('Patient Node 1: ECG Signal Transmission');

xlabel('Time (s)'); ylabel('Voltage (mV)');

legend('Raw Sensor Data', 'Received & Filtered at Gateway');

grid on;

% Plot SpO2 Signal

subplot(3, 1, 2);

plot(t, spo2_tx, 'c:', 'LineWidth', 0.8); hold on;

plot(t, spo2_rx, 'b', 'LineWidth', 1.5);

yline(95, 'r--', 'Critical SpO2 Limit (95%)');

title('Patient Node 2: SpO2 Level Monitoring');

xlabel('Time (s)'); ylabel('SpO2 (%)');

legend('Transmitted (Noisy)', 'Gateway Filtered', 'Location', 'best');

grid on;

% Plot Temperature Signal

subplot(3, 1, 3);

plot(t, temp_tx, 'm:', 'LineWidth', 0.8); hold on;

plot(t, temp_rx, 'm', 'LineWidth', 1.5);

yline(37.5, 'r--', 'Fever Threshold (37.5°C)');

title('Patient Node 3: Body Temperature Monitoring');

xlabel('Time (s)'); ylabel('Temperature (°C)');

legend('Transmitted (Noisy)', 'Gateway Filtered', 'Location', 'best');

grid on;

OUTPUT: